Weather protection grille ALAR

Specification texts

Weather protection grille type ALAR-... in round design consisting of L-formed profiled frame and horizontal, fixed, rain-repellent blades, distance 50 mm, with fastening holes and back-fitted bird protection grille made of 8x8 mm galvanised perforated plate.

Product: SCHAKO type ALAR-...

Dimensions:

-	Nominal width (-xxx)

200 - 224 - 250 - 280 -315 -355 - 400 - 450 - 500 - 560 - 630 - 710 - 800 - 900

(in mm, always with 3 digits)

No special sizes available!

Material (frame and blades) and paint

-	made of galvanised sheet steel (-SV, standard)

- without paint (-0000, not possible for -SB)

-	made of painted sheet steel (-SB)

- painted, RAL colour can be freely selected (-xxxx) (always with 4 digits) (not possible for -SV)

-	made of stainless steel 1.4301 (-V2)

- untreated, not painted (-0000, standard for -V2)

- painted, RAL colour can be freely selected (-xxxx) (always with 4 digits)

- painted in the colour sand silver (-SAND)

-	made of stainless steel 1.4571 (-V4)

- untreated, not painted (-0000, standard for -V4)

- painted, RAL colour can be freely selected (-xxxx) (always with 4 digits)

- painted in the colour sand silver (-SAND)

-	made of aluminium (-AL)

- untreated, not painted (-0000, standard for -AL)

- painted to the colour RAL 9006, white aluminium (-9006 only possible for -AL)

- painted, RAL colour can be freely selected (-xxxx) (always with 4 digits)

Bird protection grille

-	Perforated plate 8x8 mm, made of galvanised sheet steel