Membrane Absorber Model MAK / MAS

Specification texts

Membrane absorber in baffle design with completely smooth surface for use in installations with extremely high hygienic requirements to VDI 6022, VDI 2052 or in ventilation ducts with large amounts of dust or dirt or chemical stresses. Honeycomb inner-chamber made of aluminium carrier plates. Outside walls made of aluminium perforated sheet membrane foil with cover membrane foil. Baffle thickness 100 mm (-100).

Product: SCHAKO type MAK-...

-	Material

-	Aluminium (-AL) (standard)

Membrane absorber in silencer design with completely smooth surface for use in installations with extremely high hygienic requirements to VDI 6022, VDI 2052 or in ventilation ducts with large amounts of dust or dirt or chemical stresses. Inner chambers in honeycomb design made of aluminium support plates. Outside walls made of aluminium perforated sheet membrane foil with cover membrane foil. Baffle thickness 100 mm (-100).

Ducts made of aluminium with saddle joints and connecting flanges Metu profile M3.

Product: SCHAKO type MAS-...

-	Material

-	Aluminium (-AL) (standard)