Air diffuser Model ZMD

Specification texts

Air diffuser with high inductive effect, flame-resistant. Consisting of a plastic (polystyrene 4754) nozzle head with radially arranged apertures with integrated adjusting disc and connection spigot.

Product: SCHAKO type ZMD-100/-150

Model:

ZMD-...-D00	=	connection spigot for installation in ceilings (without cover plate)

ZMD-...-D01	=	connection spigot for installation in ceilings (with cover plate)

ZMD-...-K00	=	connection spigot for installation in ducts (only possible without cover plate)

ZMD-...-K01	=	connection spigot for installation in ducts with curved blade (only possible without

cover plate)

ZMD-...-RA0	=	connection spigot for installation in round ducts ø R220-280 mm (only possible

without cover plate)

ZMD-...-RA1	=	connection spigot for installation in round ducts ø R220-280 mm with curved blade

(only possible without cover plate)

ZMD-...-RB0	=	connection spigot for installation in round ducts ø R290-540 mm (only possible

without cover plate)

ZMD-...-RB1	=	connection spigot for installation in round ducts ø R290-540 mm with curved blade

(only possible without cover plate)

ZMD-...-RC0	=	connection spigot for installation in round ducts ø R550-2000 mm (only possible

without cover plate)

ZMD-...-RC1	=	for installation in round ducts ø R550-2000 mm with curved blade (only possible

without cover plate)

Fastening:

-	Screw mounting (-SM)

-	Screws must be provided on site.

Accessories:

-	Plug for air pattern adjustment (-X)

-	Sheet metal collar ø490mm (-BK1) (not possible for ceiling installation -D00 / -D01)